Guia de como configurar o BbB

Horizontal Plane

# Adjust Fideducial Delay

- Output off & FB Off ?

- Setup – SRAM

- Fideducial Signal Offset (adjustments):

  - coarse:  2 ns steps

  - fine: 870, 930, 970 ps

  - Until most signal power is in a specific bunch (aligned with bunch index from timing?)

# Finding de Beam

- Drive all bunches (!), drive 0, freq = 150 kHz, span 250 kHz

- FB timing, Sweep ADC delay with steps of 2 ns, looking at MEAN ( RMS?)  counts

- Scan horizontal phase FBE-LT and find largest p-p on MEAN count (RMS?)

- Select bunch “449” (look @ spectrum), Find the beam @ spectrum

- There is a resonance line shoing up!

- Zoom in excitation span based on where the peaks shows up

- Select auto trigger HARD, Acquisition POST trigger

- Maximize peak:

  - Sweep ADC Delay, bracket peak maximum and estimate middle point looking at the resonance: from 500 ps to 1300 ps, center ~ 900 ps

  - Sweep FBE Horizontal Phase, same algorithm: 9 - 700 ps, center ~ 350 ps

- Now, looking @ RMS count, iterate:

  - Reoptimize ADC Delay: 600 ps to 1300, center ~ 950 ps (small readjustment)

  - Maybe another iteration of FBE Horizontal Phase (ended up in the same value of 350 ps)

- Signal optimized at bunch “449”. Timing says 595. Change Fiducal Delay for matching

  - 449 – 595 = -146, -146 + 864 = 718

  - Enter 718 in fiducial delay

- Important to have a clean RMS signal (MEAN is not relevant)

- Checkinh bunch measurement coupling

  - Compare betatron peaks for bunch 959 and neighbors (shoukd be reduced by a few dBs)

# Configuration of bunch driving

- Now, intead of driving all bunches we should do a binary search to find which bunch is system is driving

- bracketing: 1-864, … , 272:286 …

- Signals @ 279 and 280 ( the system is kicking two bunches!)

- Change DAC Delay until most power is at one bunch! (1000 ps)

- Driving “279” but signal @ 594, change OUTPUT DELAY 548??

- Update driving pattern to 595

- Peak for 595 is 6 dB, for 591 is -9 dB. Diff of 14 dB. Seems ok.

- Optimize driving 595 with signal at 595 using DAC Delay

# Setup FB (for bunch 595)

- Expected tune is 0.156, 1/0.156 = 6.4 taps x2, select 13 taps

- Open coeffs panel

  - Marker @ 0.156

  - Freq = 0.156

  - taps = 13

  - Phase = 0

  - Gain = 0.05

  - No downsampling

- Check coeffs

- Enable FB !

- Look at RMS and check is signal went down with FB enabled

- Increase gain from 0.05, 0.1, 0.2 …

- Continue optimization, now without drive excitation

- Increase gain to 0.4, 0.8, 1.0 (full scale)

- We should see a notch response at spectrum

- Change Phase Degree: -40 deg (find value as to make notch symmetric)

- Increase gain shift: from 0 to 1, from 1 to 2 …

- We shoukd get a symmetric notch, shoullders should be small.

- Marker 2 set for tracking notch

- Decrease shift gain 2 → 0 (looses tracking of the notch)